|  | University of Central Punjab Faculty of Information Technology & Computer Sciences Computer Organization and Assembly Language, Fall 2025 Graded Lab 3 Ver 2 |  |
| --- | --- | --- |
|  | Total Marks: 15 | Time Allowed: 35 min only |
| Section | Student Name | Student Registration |

### Part A: Direct Stack Access (Medium - 5 Marks)

Write an 8086 assembly program that:

- Pushes five values onto the stack: 10h, 20h, 30h, 40h, 50h (in this order)

- WITHOUT using POP instructions, access the stack using BP register to:

  - Read the 2nd value from top (40h) and store in AX

  - Read the 4th value from top (20h) and store in BX

  - Calculate their sum and store in CX

Expected Output:

- AX = 0040h

- BX = 0020h

- CX = 0060h

- Stack should remain unchanged (all 5 values still on stack)

Hint: Use mov bp, sp and then access [bp+offset]

### Part B: Procedure with Register Parameters (Medium - 5 Marks)

Write a procedure multiply_and_add that:

- Receives THREE parameters through registers: AX, BX, and CX

- Calculates: (AX × 2) + (BX × 3) + CX

- Returns the result in DX

Main program should:

- Load AX = 5, BX = 10, CX = 15

- Call the procedure

- Store the result in a variable answer

Expected Result: DX = 55 (decimal) = 0037h

- Calculation: (5×2) + (10×3) + 15 = 10 + 30 + 15 = 55

### Part C: Array Processing with Stack Parameters (Medium - 5 Marks)

Write a procedure find_smallest that:

- Receives array offset and size via stack (pass by reference)

- Finds the SMALLEST element in the array

- Returns the smallest value in AL and its index position in AH

Test with array: db 45, 23, 67, 12, 89, 34 (size = 6)

Expected Result:

- AL = 12 (smallest value)

- AH = 3 (index position, 0-based)

Hint:

- Initialize a variable with first element as minimum

- Loop through array comparing each element

- Track both value and index