Lab-3 Exercise

Name: M. Hussain Durrani

Roll No: L1F24BSCS0937

Section: C-9

Task-1

Write a program that stores the following numbers into the current data segment at offset: 0x1000 onwards. The program then calculates the sum of these numbers and stores it in the DX register, then stores the value of DX at the physical location: 0xCD1F3.

Numbers: 0x1F01, 0xA0EF, 0x7704, 0x34B0, 0x2250

SOLUTION:

; Task-1

.model small

.code

start:

mov     di, 0x1000

mov     word ptr [di], 0x1F01

add     di, 2

mov     word ptr [di], 0xA0EF

add     di, 2

mov     word ptr [di], 0x7704

add     di, 2

mov     word ptr [di], 0x34B0

add     di, 2

mov     word ptr [di], 0x2250

xor     ax, ax

xor     bx, bx

clc

mov     dx, word ptr [0x1000]

add     ax, dx

adc     bx, 0

mov     dx, word ptr [0x1002]

add     ax, dx

adc     bx, 0

mov     dx, word ptr [0x1004]

add     ax, dx

adc     bx, 0

mov     dx, word ptr [0x1006]

add     ax, dx

adc     bx, 0

mov     dx, word ptr [0x1008]

add     ax, dx

adc     bx, 0

mov     dx, ax

mov     ax, 0xCD1F

mov     es, ax

mov     word ptr es:[0x0003], dx

hlt

end start

Task-2

The "HLT" has a machine code of "0xF4".

Insert this instruction before the first instruction of your program implemented in Task-1.

SOLUTION:

.model small

.stack 100h

.data

.code

main proc

hlt

mov  ax, @data

mov  ds, ax

mov  word ptr [1000h], 1F01h

mov  word ptr [1002h], A0EFh

mov  word ptr [1004h], 7704h

mov  word ptr [1006h], 34B0h

mov  word ptr [1008h], 2250h

xor  ax, ax

xor  bx, bx

mov  dx, [1000h]

add  ax, dx

adc  bx, 0

mov  dx, [1002h]

add  ax, dx

adc  bx, 0

mov  dx, [1004h]

add  ax, dx

adc  bx, 0

mov  dx, [1006h]

add  ax, dx

adc  bx, 0

mov  dx, [1008h]

add  ax, dx

adc  bx, 0

mov  dx, ax

mov  ax, 0CD1Fh

mov  es, ax

mov  word ptr es:[0003h], dx

hlt

main endp

end main

Task-3 CP

Assume that the registers have the following values (all in hex) and that CS = 1000, DS = 2000, SS = 3000, SI = 4000, DI = 5000, BX = 6080, BP = 7000, AX = 25FF, CX = 8791, and DX = 1299. Calculate the physical address of the memory.

| - MOV [SI], AL | - MOV [BX] + 30, DX |
| --- | --- |
| - MOV [SI + BX + 8], AH | - MOV [BP] + 200, AX |
| - MOV [BX], AX | - MOV [BP + SI + 100], BX |
| - MOV [DI + 6], BX | - MOV [SI]+50, AH |
| - MOV [DI][BX] + 28, CX | - MOV [DI + BP + l00], AX |

SOLVED IMAGES: